Detailed Plan for PPG/rPPG Pulse Detection in Swift iOS App

Overview

You can implement photoplethysmography (PPG) and remote photoplethysmography (rPPG) pulse detection in your SwiftUI iPhone app using the device's camera. PPG measures blood volume changes by analyzing light variations through tissue, while rPPG extracts similar signals from facial video feeds without physical contact.[1][2][3]

Architecture Options

Option 1: Contact-Based PPG (Finger on Camera)

Uses the iPhone camera with flash to detect blood volume changes when a finger covers the camera lens.[4][5][6]

Option 2: Remote PPG (Facial Detection)

Analyzes facial video to extract heart rate from subtle color changes in skin.[7][8][9]

Option 3: Hybrid Approach

Combines both methods for different use cases within your app.

Core Implementation Components

1. Camera Setup with AVFoundation

import AVFoundation

      import SwiftUI

        
      class CameraManager: NSObject, ObservableObject {
    private let captureSession = AVCaptureSession()
    private var deviceInput: AVCaptureDeviceInput?
    private var videoOutput: AVCaptureVideoDataOutput?
    private let sessionQueue = DispatchQueue(label: "camera.session.queue")
    
    @Published var currentFrame: CGImage?
    
    override init() {
        super.init()
        setupCamera()
    }
    
    private func setupCamera() {
        sessionQueue.async { [weak self] in
                    self?.configureSession()
        }
    }
    
    private func configureSession() {
        captureSession.beginConfiguration()
        
        // Configure camera input
                guard let camera = AVCaptureDevice.default(for: .video) else { return }
        
        do {
            let input = try AVCaptureDeviceInput(device: camera)
            if captureSession.canAddInput(input) {
                captureSession.addInput(input)
                deviceInput = input
            }
        } catch {
            print("Camera setup error: \(error)")
            return
                }
        
        // Configure video output
                let output = AVCaptureVideoDataOutput()
        output.setSampleBufferDelegate(self, queue: sessionQueue)
        
        if captureSession.canAddOutput(output) {
            captureSession.addOutput(output)
            videoOutput = output
        }
        
        captureSession.commitConfiguration()
    }
}

        
      extension CameraManager: AVCaptureVideoDataOutputSampleBufferDelegate {
    func captureOutput(_ output: AVCaptureOutput, didOutput sampleBuffer: CMSampleBuffer, from connection: AVCaptureConnection) {
        guard let pixelBuffer = CMSampleBufferGetImageBuffer(sampleBuffer) else { return }
        
        // Process frame for PPG analysis
                processFrameForPPG(pixelBuffer: pixelBuffer)
    }
}

2. Signal Processing Pipeline

import Accelerate

        
      class PPGProcessor: ObservableObject {
    private var redChannelBuffer: [Float] = []
    private var greenChannelBuffer: [Float] = []
    private var blueChannelBuffer: [Float] = []
    private let bufferSize = 300 // ~10 seconds at 30fps
            private let samplingRate: Float = 30.0
            
    @Published var heartRate: Double = 0.0
            @Published var signalQuality: Float = 0.0
            
    func processFrame(_ pixelBuffer: CVPixelBuffer) {
        CVPixelBufferLockBaseAddress(pixelBuffer, .readOnly)
        defer { CVPixelBufferUnlockBaseAddress(pixelBuffer, .readOnly) }
        
        let baseAddress = CVPixelBufferGetBaseAddress(pixelBuffer)
        let bytesPerRow = CVPixelBufferGetBytesPerRow(pixelBuffer)
        let width = CVPixelBufferGetWidth(pixelBuffer)
        let height = CVPixelBufferGetHeight(pixelBuffer)
        
        // Extract RGB averages
                let rgbAverages = calculateRGBAverages(
            baseAddress: baseAddress,
            width: width,
            height: height,
            bytesPerRow: bytesPerRow
        )
        
        // Add to circular buffers
                updateBuffers(red: rgbAverages.red, green: rgbAverages.green, blue: rgbAverages.blue)
        
        // Process signal if buffer is full
                if redChannelBuffer.count >= bufferSize {
            processSignal()
        }
    }
    
    private func calculateRGBAverages(baseAddress: UnsafeMutableRawPointer?, width: Int, height: Int, bytesPerRow: Int) -> (red: Float, green: Float, blue: Float) {
        guard let baseAddress = baseAddress else { return (0, 0, 0) }
        
        var redSum: Float = 0
                var greenSum: Float = 0
                var blueSum: Float = 0
                let pixelCount = width * height
        
        for y in 0..<height {
            let rowPtr = baseAddress.advanced(by: y * bytesPerRow)
            for x in 0..<width {
                let pixelPtr = rowPtr.advanced(by: x * 4) // Assuming BGRA format
                        let blue = pixelPtr.load(as: UInt8.self)
                let green = pixelPtr.advanced(by: 1).load(as: UInt8.self)
                let red = pixelPtr.advanced(by: 2).load(as: UInt8.self)
                
                redSum += Float(red)
                greenSum += Float(green)
                blueSum += Float(blue)
            }
        }
        
        return (
            red: redSum / Float(pixelCount),
            green: greenSum / Float(pixelCount),
            blue: blueSum / Float(pixelCount)
        )
    }
    
    private func processSignal() {
        // Use green channel for PPG (most sensitive to blood volume changes)
                let filteredSignal = applyBandpassFilter(to: greenChannelBuffer)
        let heartRate = calculateHeartRate(from: filteredSignal)
        
        DispatchQueue.main.async {
            self.heartRate = heartRate
        }
    }
    
    private func applyBandpassFilter(to signal: [Float]) -> [Float] {
        // Bandpass filter for heart rate range (0.75 - 3.0 Hz, 45-180 BPM)
                let lowCutoff: Float = 0.75
                let highCutoff: Float = 3.0
                
        // Implementation using vDSP for efficient filtering
                var filteredSignal = signal
        
        // Apply Butterworth bandpass filter
                // This is a simplified version - you might want to use a proper DSP library
                return butterworthBandpass(signal: filteredSignal, 
                                 lowFreq: lowCutoff, 
                                 highFreq: highCutoff, 
                                 sampleRate: samplingRate)
    }
    
    private func calculateHeartRate(from signal: [Float]) -> Double {
        // FFT-based heart rate calculation
                let fft = performFFT(on: signal)
        let dominantFrequency = findDominantFrequency(in: fft)
        return Double(dominantFrequency * 60) // Convert to BPM
            }
}

3. Open Source Integration Options

Option A: rPPG-Toolbox Integration

The rPPG-Toolbox provides state-of-the-art algorithms but is Python-based. You can:[9][7]

- Create a Swift wrapper around the Python models

- Port key algorithms to Swift for iOS

- Use Core ML to run converted models

import CoreML

        
      class rPPGModel {
    private var model: MLModel?
    
    init() {
        loadModel()
    }
    
    private func loadModel() {
        guard let modelURL = Bundle.main.url(forResource: "PhysNet", withExtension: "mlmodel"),
              let loadedModel = try? MLModel(contentsOf: modelURL) else {
            print("Failed to load rPPG model")
            return
                }
        self.model = loadedModel
    }
    
    func predict(videoFrames: [CVPixelBuffer]) -> [Float]? {
        // Convert frames to model input format
                // Run inference
                // Return PPG signal
                return nil
            }
}

Option B: Swift Waveform Plotter

Use the swift-waveform_plotter package for real-time signal visualization:[10]

// Package.swift
        dependencies: [
    .package(url: "https://github.com/maurovm/swift-waveform_plotter", from: "1.0.0")
]

4. SwiftUI User Interface

import SwiftUI

        
      struct PPGView: View {
    @StateObject private var cameraManager = CameraManager()
    @StateObject private var ppgProcessor = PPGProcessor()
    @State private var isRecording = false
            
    var body: some View {
        VStack {
            // Camera preview for finger placement
                    CameraPreviewView(session: cameraManager.captureSession)
                .frame(height: 200)
                .cornerRadius(10)
            
            // Heart rate display
                    VStack {
                Text("Heart Rate")
                    .font(.headline)
                Text("\(Int(ppgProcessor.heartRate))")
                    .font(.largeTitle)
                    .fontWeight(.bold)
                Text("BPM")
                    .font(.caption)
            }
            .padding()
            
            // Signal quality indicator
                    ProgressView("Signal Quality", value: ppgProcessor.signalQuality)
                .padding()
            
            // Recording controls
                    Button(action: toggleRecording) {
                Text(isRecording ? "Stop" : "Start Measuring")
                    .foregroundColor(.white)
                    .padding()
                    .background(isRecording ? Color.red : Color.blue)
                    .cornerRadius(10)
            }
        }
        .onAppear {
            cameraManager.requestPermissions()
        }
    }
    
    private func toggleRecording() {
        if isRecording {
            cameraManager.stopSession()
        } else {
            cameraManager.startSession()
        }
        isRecording.toggle()
    }
}

5. Flash Control for PPG

extension CameraManager {
    func enableTorch(_ enabled: Bool) {
        guard let device = deviceInput?.device,
              device.hasTorch else { return }
        
        do {
            try device.lockForConfiguration()
            if enabled && device.isTorchModeSupported(.on) {
                try device.setTorchModeOn(level: 1.0)
            } else {
                device.torchMode = .off
            }
            device.unlockForConfiguration()
        } catch {
            print("Torch control error: \(error)")
        }
    }
}

Recommended Implementation Strategy

Phase 1: Basic PPG (Contact-based)

- Set up camera capture with AVFoundation[11][12]

- Implement RGB channel extraction from camera frames[13][14]

- Create basic bandpass filtering for heart rate frequencies

- Add real-time signal processing with simple peak detection

Phase 2: Enhanced Processing

- Integrate advanced filtering using vDSP/Accelerate framework

- Add signal quality assessment and noise reduction

- Implement FFT-based frequency analysis for accurate heart rate

- Create user feedback system for proper finger placement

Phase 3: rPPG Implementation

- Add face detection using Vision framework

- Implement region of interest (ROI) tracking for stable signal extraction

- Port key algorithms from rPPG-Toolbox or create Core ML models[7]

- Add motion compensation and artifact rejection

Phase 4: Production Features

- Implement data persistence with HealthKit integration

- Add trend visualization using Swift Charts

- Create export functionality for health data

- Optimize for different iPhone models and lighting conditions

Key Technical Considerations

Performance Optimization

- Use efficient pixel buffer processing  to avoid frame drops[15][13]

- Implement proper memory management for continuous video processing

- Optimize signal processing algorithms using Accelerate framework

- Consider background processing for non-critical computations

Accuracy Factors

- Proper finger placement guidance for contact PPG[6][4]

- Motion artifact detection and rejection[8][9]

- Adaptive filtering based on signal quality assessment

- Calibration against known reference devices for validation

Hardware Considerations

- iPhone camera specifications vary by model[16]

- Flash LED characteristics affect PPG signal quality

- Ambient light compensation for rPPG implementations

- Frame rate optimization (30 FPS recommended for PPG)

This implementation provides a solid foundation for PPG/rPPG pulse detection in your SwiftUI iPhone app, with options for both contact-based and remote measurement approaches using open-source components and efficient iOS frameworks.

⁂

- https://www.helixapps.co.uk/blog/apple-watch-photoplethysmography-ppg

- https://pmc.ncbi.nlm.nih.gov/articles/PMC9267568/

- https://www.rouast.com/blog/articles/understanding-remote-photoplethysmography/

- https://apps.apple.com/us/app/instant-pulse-blood-pressure/id1569932522

- https://apps.apple.com/us/app/heartcam-heart-rate-monitor/id6743995535

- https://pubmed.ncbi.nlm.nih.gov/25626548/

- https://github.com/ubicomplab/rPPG-Toolbox

- https://openaccess.thecvf.com/content/CVPR2025/papers/Shao_Remote_Photoplethysmography_in_Real-World_and_Extreme_Lighting_Scenarios_CVPR_2025_paper.pdf

- https://ubicomplab.cs.washington.edu/pdfs/rppg-toolbox.pdf

- https://github.com/maurovm/swift-waveform_plotter_example

- https://www.appcoda.com/avfoundation-swift-guide/

- https://stackoverflow.com/questions/28756363/how-to-capture-picture-with-avcapturesession-in-swift

- https://clouddevs.com/ios/exploring-core-video/

- https://blog.jacobstechtavern.com/p/advanced-core-image

- https://www.createwithswift.com/camera-capture-setup-in-a-swiftui-app/

- https://pmc.ncbi.nlm.nih.gov/articles/PMC8347217/

- https://iosapptemplates.com/blog/top-swift-libraries-bootstrap-ios/

- https://academic.oup.com/europace/article/26/4/euae065/7648812

- https://blog.arrowhitech.com/10-outstanding-open-source-swift-libraries/

- https://aglowiditsolutions.com/blog/ios-open-source-libraries/

- https://forums.swift.org/c/related-projects/25

- https://www.nature.com/articles/s43856-024-00519-6

- https://happitech.com/our-technology/

- https://haconvention2025.dryfta.com/abstract-archive/abstract/public/176/contactless-assessment-of-patient-vital-signs-using-remote-photoplethysmography-in-the-emergency-department-capture-1-project

- https://www.reddit.com/r/SwiftUI/comments/1ae95ro/where_are_all_the_swift_ui_component_libraries/

- https://www.mindbreathapp.com/ppg-heartrate-mobile-hrv

- https://www.philips.com/a-w/about/innovation/ips/ip-licensing/programs/biosensing-by-rppg.html

- https://github.com/ppgco/ppg-core-ios-sdk

- https://www.youtube.com/watch?v=v-8OEviEZ4c

- https://engineeringportal.nielsen.com/wiki/Digital_Measurement_iOS_Swift_Package_Manager_Guide

- https://betterprogramming.pub/measuring-your-heart-rate-using-your-phones-camera-and-flutter-f444d3c4272a

- https://neurips.cc/virtual/2023/poster/73458

- https://www.youtube.com/watch?v=aDoYHak5UsA

- https://stackoverflow.com/questions/27629636/how-to-implement-avcam-for-swift

- https://openreview.net/forum?id=q4XNX15kSe&noteId=rpGqLYsUsZ

- https://developer.apple.com/videos/play/wwdc2018/411/

- https://www.youtube.com/watch?v=hg-6sOOxeHA

- https://arxiv.org/abs/2210.00716

- https://www.youtube.com/watch?v=al2O8uCFD4E

- https://developer.apple.com/documentation/sensorkit/srsensor/photoplethysmogram

- https://proceedings.neurips.cc/paper_files/paper/2023/hash/d7d0d548a6317407e02230f15ce75817-Abstract-Datasets_and_Benchmarks.html

- https://docs.swift.org/swiftpm/documentation/packagemanagerdocs/

- https://developer.apple.com/documentation/sensorkit/srphotoplethysmogramsample

- https://ubicomplab.cs.washington.edu/publications/rppg-toolbox/

- https://apps.apple.com/us/app/swap-rgb/id1109935965

- https://www.reddit.com/r/iosapps/comments/1m1c709/solo_dev_journey_how_i_built_an_iphone/

- https://www.reddit.com/r/apple/comments/304n3a/apple_invents_3sensor_iphone_camera_with_light/

- https://github.com/richrd/heart-rate-monitor

- https://developer.apple.com/videos/play/wwdc2025/253/

- https://www.ignaciomellado.es/blog/Measuring-heart-rate-with-a-smartphone-camera

- https://developer.apple.com/documentation/avfoundation/capturing-still-and-live-photos

- https://stackoverflow.com/questions/64658325/cifilters-for-compositing-red-green-and-blue-color-channels

- https://stackoverflow.com/questions/14441711/ios-detect-heart-rate

- https://www.objc.io/issue-21/camera-capture-on-ios.html

- https://developer.apple.com/documentation/appkit/nscolor/init(red:green:blue:alpha:)

- https://apps.apple.com/us/app/facial-heart-rate-lite/id912136772

- https://www.youtube.com/watch?v=ZYPNXLABf3c

- https://stackoverflow.com/questions/19773631/ios-heart-rate-detection-algorithm

- https://www.reddit.com/r/iOSProgramming/comments/1l2hgvx/how_to_animate_smooth_camera_lens_transition_wide/

- https://welltory.com/devices/iphone-heart-rate-app/

- https://juniperphoton.substack.com/p/workaround-for-tricky-issues-when

- https://www.reddit.com/r/GIMP/comments/6dxsld/is_there_an_easy_way_separate_an_images_rgb/

- https://www.youtube.com/watch?v=GhQUMCmCISU

- https://www.objc.io/issues/23-video/core-image-video

- https://forums.swift.org/t/asyncstream-for-cmsamplebuffer-in-camera-delegate-ensuring-smooth-performance-across-devices/76974

- https://stackoverflow.com/questions/12520081/core-image-filters-to-apply-curves-to-each-rgb-channel-separately

- https://developer.apple.com/documentation/corevideo/cvpixelbuffer-q2e

- https://stackoverflow.com/questions/36313943/sample-buffer-delegate-swift-2-for-real-time-video-filter

- https://developer.apple.com/library/archive/documentation/GraphicsImaging/Reference/CoreImageFilterReference/

- https://stackoverflow.com/questions/11655545/how-do-i-analyze-video-stream-on-ios

- https://developer.apple.com/documentation/Vision/recognizing-objects-in-live-capture

- https://betterprogramming.pub/handling-core-image-filter-processing-with-concurrency-in-swift-a016396c3a07

- https://developer.apple.com/documentation/vision/pixelbufferobservation

- https://noahgilmore.com/blog/cifilter-colorwheel

- https://www.createwithswift.com/getting-started-with-core-image/

- https://heartbeat.comet.ml/how-to-capture-the-best-frame-in-an-ios-image-processing-app-5a14829a03f1

- https://gist.github.com/JoshuaSullivan/b0e39f9009e44063b366cdc46773601e