### DualPath

### https://arxiv.org/pdf/2602.21548

### Summary

DualPath rethinks KV-Cache loading for agentic, multi-turn LLM inference by adding a second read path: storage→decode→RDMA→prefill, in addition to the conventional storage→prefill path. This design aggregates storage NIC bandwidth across decode and prefill engines, routes all GPU I/O through the compute NIC (CNIC) with QoS isolation, and uses a two-level scheduler to choose read paths and pack layerwise prefill to overlap I/O with computation. The system reports up to 1.87× offline throughput and ~1.96× online serving throughput improvements versus a Basic baseline. arxiv.org

### Technical Evaluation

Core idea and novelty

- Dual read paths. Introducing a storage→decode path that then transfers KV blocks to prefill engines over the compute fabric is a simple but powerful reallocation of otherwise idle storage bandwidth on decode nodes. This reframes storage I/O from a single saturated resource into a pooled, schedulable capacity. arxiv.org

System design and mechanisms

- CNIC-centric transfers and QoS. All GPU I/O is routed through the CNIC and mapped to virtual lanes so latency-sensitive collectives remain protected while KV-Cache transfers use low-priority lanes. This is a pragmatic way to leverage existing RDMA QoS features to avoid interference. arxiv.org

- Two-level scheduler. The inter-engine scheduler balances token counts and disk read queues to pick PE vs DE reads; the intra-engine scheduler uses a compute-quota packing strategy and chunked prefill to reduce synchronization bubbles. These heuristics are well aligned with the observed bottlenecks. arxiv.org

- Layer and buffer layout. Small DRAM PE/DE buffers plus Full Block and Layer Block layouts enable layerwise streaming and overlap of transfers with computation while keeping HBM pressure manageable. arxiv.org

Implementation realism

- The authors implemented DualPath on an in-house stack using GPUDirect RDMA, CNIC doorbell batching, and a 3FS storage backend; the implementation required modest code changes (~5K LOC). This demonstrates practical deployability on modern InfiniBand/RDMA clusters. arxiv.org

### Strengths

- Problem grounded in real workloads. The paper convincingly shows agentic workloads have very high KV-Cache hit rates and extreme context lengths, making storage I/O the dominant bottleneck. arxiv.org

- High practical value. DualPath reuses existing hardware features (RDMA, NIC QoS) and requires modest software changes, increasing the likelihood of adoption in production datacenters. arxiv.org

- Comprehensive evaluation. The authors evaluate offline (rollout) and online serving, multiple model sizes (including a 660B MoE model), and scale experiments up to 1,152 GPUs, showing near-linear scaling and consistent gains. arxiv.org

- Ablations and diagnostics. The paper isolates contributions from layerwise prefill, dual-path loading, and scheduling, and reports load-balance improvements for storage NICs and attention execution times. arxiv.org

### Weaknesses and Risks

- Working set growth under realistic gaps. The analysis shows working set size grows with arrival rate and real-world tool call gaps; at high arrival rates this can exceed memory budgets and reduce cache hit rates, eroding benefits. The paper acknowledges this but leaves hierarchical caching and eviction strategies for future work. arxiv.org

- Heuristic scheduler tuning. The two-level scheduler depends on thresholds (e.g., read queue α, token limit β) and P/D ratios; automated, workload-adaptive tuning is not fully explored and may be necessary for heterogeneous deployments. arxiv.org

- Hardware dependency. Benefits rely on RDMA-capable CNICs, GPUDirect, and NIC QoS; environments lacking these features (some clouds or older clusters) will see reduced gains or require additional engineering. arxiv.org

- Limited failure and robustness analysis. The paper could expand on behavior under node failures, transient storage slowdowns, or compute-network congestion and describe fallbacks that gracefully degrade to direct reads. arxiv.org

### Recommendations and Future Work

- Adaptive scheduling. Replace static thresholds and P/D presets with online control or reinforcement learning to adapt to changing arrival patterns and storage variability. arxiv.org

- Hierarchical caching integration. Combine DualPath with a DRAM or NVMe cache layer and smarter eviction policies to contain working set growth under bursty or high-arrival workloads. arxiv.org

- Robustness engineering. Add explicit fallbacks for storage slowdowns and node failures, and evaluate how DualPath behaves under degraded network/storage conditions. arxiv.org

- Broader portability tests. Validate on diverse interconnects (RoCE, cloud NICs) and storage backends to quantify portability and required configuration changes. arxiv.org

### Verdict

DualPath is a strong, practical systems contribution that identifies a real and growing bottleneck for agentic LLM inference and offers a deployable solution with measurable gains. Its combination of a second KV-Cache read path, CNIC-centric traffic isolation, and workload-aware scheduling yields substantial throughput and utilization improvements while remaining implementable on modern datacenter hardware. The paper balances design, implementation, and evaluation well and opens clear, actionable directions for adaptive scheduling and hierarchical caching to make the approach robust in broader production settings. arxiv.org

If you want, I can (pick one):

- Convert this review into a one-page peer-review with pros/cons and suggested reviewer questions; or

- Extract and annotate the paper’s key figures and tables (I’ll cite the tab for each figure); or

- Draft a short rebuttal emphasizing the strongest experimental evidence and addressing likely reviewer concerns. Which would you prefer?